- C-DAC Placement Papers

- Select the Best Choice(s):

- 1. A set of rules, which enable orderly exchange of information between two devices.

- a) Topology b) Protocol c) Transmission Media d) None of the above

- 2. ISO stands for

- a) International Standards Organization

- b) International Organization for Standardization

- c) Both of above

- d) None of above

- 3. Proposal for a new Internet standard is called

- a) RFC b) Internet draft c) Draft Standard d) Proposed Standard

- 4. The data sent between layers is called

- a) Protocol Data Unit b) Datagram c) Service Data Unit d) Packet

- 5.The Private Automatic Branch Exchange is an example of

- a) Bus Topology b) Ring Topology c) Tree Topology d) Star Topology

- 6. Topology, which connects every single node in the network to every other node, is

- a) Star b) Ring c) Mesh d) Bus

- 7. Which of the following is not a hybrid topology

- a) Tree b) String c) Bus d) None of the above

- 8. The term 10Base2 indicates a network

- a) 10 mbps speed, Base Band Signaling, 200 meters segment length

- b) 10 mbps speed, Base Band Signaling, 200 meters maximum length

- c) 10 mbps speed, Base Band Signaling, 185 meters segment length

- d) None of the above

- 9. Coaxial cable, UTP cable, STP cable and fibre cables are popular network media today. Order these

- in increasing order of data security

- a) Coaxial, UTP, STP, Fibre

- b) UTP, Coaxial, STP, Fibre

- c) UTP, STP, Coaxial, Fibre

- d) None of the above

- 10. Bridges are simple devices that are used to connect LANs of

- a) Different Architecture (eg. Ethernet to Token Ring)

- b) Same architecture (eg. Ethernet to Ethernet)

- c) Architecture does not matter

- e) None of the above

- 11. Which application uses existing CSMA/VD over existing twisted-pair cable with bandwidths of

- 100 mbps?

- a) 10BaseF b) 100BaseX c) 100VG-Any

- LAN d) 10BaseFD

- 12. What model divides the network communication process into seven layers?

- a) Layered Model b) TCP/IP c) OSI d) X.25

- 13. B-ISDN uses fiber as a transmission medium and _______ as the switching infrastructure

- a) Ethernet b) FDDI c) ATM d) None of the above

- 14. ______ is the only standards based technology that has been designed from the beginning to

- accommodate the simultaneous transmission of voice, video and data

- a) Home cable network b) Satellite TV c) ATM d) All of the above

- 15. Which of the following common LAN protocols extend to OSI network layer.

- a) NetBIOS b) NetBEUI c) TCP/IP d) All of the above

- 16. Portability standards are discussed widely in four areas, which of the following is not one of them.

- a) Operating Systems

- b) Data Management

- c) Programming Languages

- d) Hardware

- 17. Which of the following is not an essential feature of Data Link Layer protocols as defined by ISO/

- OSI model

- a) Message orientation

- b) Error Detection

- c) Error correction by re-transmission

- d) None of the above

- 18. Which of the following describes the OSI Class 3 transport (TP3)

- a) Do nothing

- b) Signaled Error Recovery

- c) Multiplexing

- d) Signaled error recovery and multiplexing

- 19. The issue of checkpoint and synchronization is handled by which layer

- a) Session Layer b) Transport Layer c) Presentation layer d) Application Layer

- 20. In the OSI model, the responsibility for negotiating the encodings to be used in any perticular

- connection is entrusted upon

- a) Application Layer b) Session Layer c) Presentation Layer d) None of the above

- 21. TCP/IP is built on _________ technology

- a) Connection oriented b) Connection less c) Datagram d) Ethernet

- 22.IP addresses are ____ bit long

- a) 4 b) 64 c) 48 d) 32

- 23. Which of the following is not a network operating system

- a) Windows for Workgroup over MSDOS

- b) Novell Netware

- c) Windows 95

- d) None of the above

- 24. Fiber optic token ring networks operate at the speed of

- a) 16 mbps b) 100 mbps c) 1000 mbps d) 10 mbps

- 25. The Synchronous Optical Network is an ANSI standard. A similar standard established in Europe is

- a) Synchronous Digital Hierarchy

- b) SONET

- c) Switched Multi megabit Data Service

- d) Distributed Queue Duel Bus

- 26. In the TCP/IP protocol family ___________ provides reliable transport service.

- a) Transport Protocol b) Transport Layer c) TCP d)All of above

- 27. IP address is a ___________ address.

- a) Network Layer Address b) Layer 2 address c) Hardware Address d) None of above

- 28. An IP address when logically ANDed with netmask, the result is _________________.

- a) Host Address b) Network Address c) Broadcast Address d) None of the above

- 29. Remote boot uses _______ protocol to discover IP address of diskless machine.

- a) ICMP b) SNMP c) ARP d) RARP

- 30. Once a datagram is fragmented in a IP network it is reassembled only at ____________.

- a) Next hope b) Next Router c) Final Destination d) Never

- 31. The source quench _________ message is used to control the rate at which datagrams are

- transmitted.

- a) IP b) ICMP c) SNMP d) TCP

- 32. TCP establishes an end to end _______________ between the sender and receiver.

- a) Connection b) Virtual Circuit c) Path d) None of above

- 33. UDP is used with ___________________ Protocol.

- a) Trivial File Transfer b) ICMP c) LDAP d) All of above

- 34. IP is a ___________ protocol

- a) Routing Protocol b) Routed Protocol c) Both of above d) None of above

- 35. ______ is a complex data structure that contains details about a connection

- a) Socket b) Port c) TCB d) None of above

- 36. The bind socket call is used to __________________________

- a) Bind a local application to a remote application

- b) Bind a socket to local port

- c) Bind a socket to a remote port

- d) Bind a local port to a remote port

- 37. Each SNMP managed object belongs to a ______________

- a) Community b) Network c) Organization D) SNMP Group

- 38. RTCP is the ________________ protocol designed to work with RTP

- a) Application b) Control c) Network d) Transport

- 39. For block devices all I/O occurs through the _________________

- a) Blocks b) Device c) Buffer cache d) None of above

- 40. _______ is a international standard file format for describing interactive 3D multimedia on the

- Internet.

- a) DHTML b) XML c) VRML d) None of above

- 41. ___________ is a distance-vector routing protocol

- a) RIP b) IGRP c) OSPF d) All of above

- 42. IPng provides security through Authentication Headers and the default encryption method is

- ________

- a) SSL b) Public Key c) MD5 d) None of above

- 43. RTSP is a ___________ protocol similar to HTTP

- a) Network Layer b) Internet c) Application Layer d) None of above

- 44. OSPF features include __________________.

- a) Multi-Path routing

- b) Equal-cost

- c) Routing based on upper-layer TOS request

- d) All of above

- 45. IP address is assigned to a __________.

- a) Network b) Host c) Interface d) All of above

- 46. Token Ring network operate at a speed of

- a) 10 mbps b) 100 mbps c) 20 mbps d) 16 mbps

- 47. Ethernet is an access method that strictly adheres to the

- a) CSMA/CD b) Token Passing c) SPX/IPS d) TCP/IP

- 48. 100BASET4 operates at 100 mbps using base band signaling and the media is

- a) Two stands of fiber

- b) Two pairs of data grade UTP

- c) Four pairs of telephone grade UTP

- d) None of the above

- 49. Length of Ethernet address is

- a) 24 bit b) 12 bit c) 48 bit d) 32 bit

- 50. Frame Relay protocol is used for

- a) WAN b) LAN c) Token Ring networks d) None of the above

- 51. Distance of a Radio Link is limited by

- a) Line of site b) Capacity of the HUB device c) Both A and B d) None of the above

- 52. Basic Rate ISDN service provides

- a) 23B+1D Channels b) 2B+1D Channels c) 30B + 1D Channel d) None of the above

- 53. X.25 networks work at the maximum speed of

- a) 33.6 kbps b) 128 kbps c) 64 kbps d) 2.1 mbps

- 54. Unit for data at Transport layer is

- a) Segment b) Packet c) Frame d) Bits

- 55. A physical layer address of a node is

- a) Always fixed

- b) Changes if the NIC hardware is changed

- c) Assigned by the administrator of the node

- d) None of the above

- 56. Path determination occurs at

- a) Data Link Layer b) Session Layer c) Transport Layer d) Network Layer

- 57. One of the following is not a function of Transport Layer.

- a) Windowing b) Addressing c) Multiplexing d) Flow control

- 58. Network layer communicates path information using

- a) ICMP b) SNMP c) Some form of routing information protocol d) None of the above

- 59. Applications running on a single host are identified by service access points at Transport layer and

- Session layer interface. These SAPs are known as

- a) Ports b) Sockets c) Both A and B d) None of the above

- 60. Route poisoning technique is used to avoid

- a) Congestion problem b) Count to infinity problem c) Traffic shaping d) None of the above

- 61. The Transport Layer provide

- a) Best effort end to end packet delivery service

- b) Connection oriented end to end packet delivery service

- c) Connection oriented, reliable, end to end packet delivery service

- d) None of the above

- 62. Application data is converted in a form suitable for transmission on the network by

- a) Network Layer b) MAC Layer c) Session Layer d) Presentation Layer

- 63. Version 4 Internet Protocol uses

- a) 32 bit flat address scheme

- b) 32 bit hierarchical address scheme

- c) 64 bit address

- d) 128 bit address

- 64. Mapping between Internet Protocol address and MAC layer address are resolved using

- a) RARP b) RIP c) ARP d) TCP/IP

- 94. A host has an IP address of 192.168.6.209 and a net mask of 255.255.255.0. Which one of the

- following statements is true?

- a) The subnet number is 192.168.6.0, and the host number is 209.

- b) The subnet number is 192.168.6.192, and the host number is 17.

- c) The subnet number is 192.168.6.200, and the host number is 9.

- d) The subnet number is 192.168.6.208, and the host number is 1.

- 95. Which of the following protocols is used for network management and monitoring?

- a) X.500 b) SNMP c) SMTP d) X.400

- 96. Your network segment has a default gateway of 205.222.45.126 and a subnet mask of

- 255.255.255.192. Which of the following is true?

- a) The valid range for hosts is 205.222.1.126 through 205.222.44.126 and all the hosts must use the

- same subnet mask.

- b) The valid range for hosts is 205.222.45.126 through 205.222.45.126 and all the hosts must increase

- the subnet mask by one.

- c) The valid range for hosts is 205.222.45.65 through 205.222.45.125 and all the hosts must use the

- same subnet mask.

- d) The valid range for hosts is 205.222.1.65 through 205.222.45.126 and all the hosts must use the

- same subnet mask.

- 97. When developers write TCP/IP software, they have two mechanisms for transporting data from

- applications: TCP and UDP. Why, in many cases, is UDP used as the protocol for communications?

- a) UDP has a smaller packet size, and thus can be sent around the network with more speed.

- b) TCP has no mechanism to support short, choppy bursts of data, which some applications tend to

- produce.

- c) UDP is never used for applications. It is simply a routing protocol.

- d) UDP provides reliable delivery acknowledgements, which many applications need for success.

- 98. In a token-passing network, what can each and every device on the network be described as

- behaving like?

- a) A proxy. b) A router. c) A bridge. d) Repeater

- 99. You have become increasingly concerned with the activity at the Logical Link Layer of the

- IPX/SPX stack on a certain workstation. What data form should you be concentrating on at this level?

- a) Frames b) Packets c) Datagrams d) Bits

- 100. TCP/IP Protocol Architecture was designed to carry data over the ARPANET. What type of

- network is the ARPANET?

- a) Message switching network.

- b) Packet switching network.

- c) Packet routing network.

- d) A hybrid of a packet and message routing network.

- 101. Your company has been assigned a Class B IP address. You are administrating a country-wide

- network with 45 individual subnets. Over the next year, you will be adding 60 subnets. What subnet

- mask should you use that will allow the most hosts per subnet?

- a) 255.255.252.0 b) 255.255.254.0 c) 255.0.0.0 d) 255.255.0.0

- 102. What layer of the OSI model handles such issues as file access and transfer and virtual terminal

- emulation?

- a) Application b) Network c) Transport d) Session

- 103. You have implemented RSA encryption technology on your network. It seems to be working well.

- However, one station always receives encrypted emails as garbled text, meaning the protocol stack is

- corrupted. What OSI layer above can be assumed to be faulty?

- a) Data Link Layer b) Presentation Layer c) Session Layer d) Transport Layer

- 104. What is the maximum segment length for 10Broad36?

- a) 85 meters b) 200 meters c) 36 meters d) 3600 meters

- 105. Which networking standard describes cabling requirements for Ethernet?

- a) 802.2 b) 802.3 c) 802.4 d) 802.5

- 106. You have been given the Class C address of 205.222.5.0. What size subnet mask will give you the

- greatest possible number of hosts?

- a) An 8-bit mask of 255.255.255.255 for 254 hosts.

- b) An 8-bit mask of 255.255.255.0 for 254 hosts.

- c) An 8-bit mask of 255.255.0.0 for 65,534 hosts.

- d) An 8-bit mask of 255.255.0.0 for 8190 hosts.

- 107. Describe a fully-meshed star network.

- a) A network with a central router that has a unique path to each end-point router and some of the endpoint

- routers have links to one another.

- b) A network with a central router that feeds a number of smaller routers that in turn feed a number of

- end-point routers.

- c) A network with a central router that has a unique path to each end-point router.

- d) A network in which all the routers have unique paths to one another

- 108. Why is SNMP considered lacking in security?

- a) SNMP devices send messages about their failing conditions.

- b) The SNMP manager broadcasts sensitive information to the managed devices.

- c) The SNMP passwords are clear ASCII text.

- d) SNMP passwords cannot be changed.

- 109. Which one of the following statements correctly describes differences between a bridge and a

- layer two switch?

- a) A layer two switch is a high speed, multi-port bridge.

- b) A layer two switch cannot use the Spanning Tree Protocol to learn a network’s topology, while

- c) A layer two switch can support multiple network media types, such as twisted pair, fiber, and coax,

- while a bridge is limited to supporting a single network media type.

- d) A layer two switch is than an Ethernet hub that supports different speeds on different ports, while a

- bridge requires all ports to have the same speed.

- 110. How are packet forwarding decisions made in an OSI reference model layer three device?

- a) The device compares the destination MAC address to an internal table that associates MAC

- addresses to particular ports.

- b) In a layer three device, every packet is sent out of every port.

- c) In a layer three device, packets are forwarded based on node name.

- d) The device compares the destination network address to an internal table that associates network

- addresses to particular ports.

- 111.Which one of the following describes the IP address 192.4.2.4

- a) A Class A address that is 32 bits in length

- b) A Class B address that is 16 bits in length

- c) A Class B address that is 32 bits in length

- d) A Class C address that is 32 bits in length

- 112. What is the main function of a router?

- a) Setting up communications between networks that use dissimilar IP segments but the same Subnet

- Masks

- b) Connecting logically separate network segments.

- c) Providing IP configuration to computers using DHCP

- d) Authenticating users to the network

- e) Connecting to the Internet.

- 113. Network cards in computers communicate with each other using unique MAC addresses, similar

- to street addresses. Where do these unique addresses come from?

- a) The network administrator makes up addresses to use and then programs them into the software.

- b) The manufacturer publishes a range of addresses in the manual that you choose from.

- c) The network card searches the network and then assigns itself a unique address from an elaborate

- algorithm using the alread-allocated addresses.

- d) The IEEE assigns every manufacturer a prefix and a block of address range, typically expressed in

- hexadecimal bridge can.