- DRDO Aptitude Paper Sept 2008

- I have attended Scientist Entry Test on 7th Sep. Hereby I am submitting paper for Electronics and

- Communication branch…….. Hope it helps future aspirants……..

- Section- A

- 1.The current I in the given network.

- a) 1A b) 3A c) 5A d) 7A

- 2.For the Delta- Wye transformation in given figure, the value of the resistance R is.

- a) 1/3 ohms b) 2/3 ohms c) 3/2 ohms d) 3 ohms

- 3.In the given network, the Thevenin’s equivalent as seen by the load resistance Rl is

- a) V=10 V, R= 2ohms b) V=10V, R=3 ohms c) V=15V, R= 2ohms d) V=15V, R=3 ohms

- 4.The current I in a series R-L circuit with R=10 ohms and L=20mH is given by i=2sin500t A. If v is

- the voltage across the R-L combination then i

- a) lags v by 45 degree b) is in-phase with v c) leads v by 45 d) lags v by 90

- 5.In through given network, the mesh current I and the input impedance seen by the 50 V source,

- respectively, are

- a) 125/13 A and 11/8 ohms b) 150/13 A and 13/8 ohms c) 150/13 A and 11/8 ohms d) 125/13 A

- and 13/8 ohms

- 6.A voltage source having a source impedance Z = R + jX can deliver maximum Average power to a

- load impedance Z, when

- a) Z = R + jX b) Z = R c) Z = jX d) Z = R –jX

- 7.In the given circuit, the switch S is closed at t=0. Assuming that there is no initial Charge in the

- capacitor, the current i(t) for t>0 is

- a) V/R e^ (-2t/RC) b) V/R e^ (-t/RC) c) V/2R e^ (-2t/RC) d) V/2R e^ (-t/RC)

- 8.For the circuit in given figure, if e(t) is a ramp signal, the steady state value of the Output voltage v(t)

- is

- a) 0 b) LC c) R/L d) RC

- 9.For the series RLC circuit in given figure, if w=1000 rad/sec, then the current I (in Amperes) is

- a) 2 ∟-15 b) 2 ∟15 c) √2∟-15 d) √2∟15

- 10.The Y-parameter matrix (mA/V) of the two-port given network is

- a) [2 -1 -1 2] b) [2 1 -1 2] c) [1 -2 -1 2] d) [2 1 1 2]

- 11.The maximum number of trees of the given graph is

- a) 16 b) 25 c) 100 d) 125

- 12.Given figure shows a graph and one of its trees. Corresponding to the tree, the group of branches

- that CAN NOT constitute a fundamental cut set is

- a) 1,2,3 b) 1,4,6,8,3 c) 5,6,8,3 d) 4,6,7,3

- 13.The Y-parameter matrix of a network is given by Y=[1 1 -1 1] A/V. The Z11 parameter of the same

- network is

- a) ½ ohms b) 1/√2 ohms c) 1 ohms d) 2 ohms

- 14.For the given circuit, the switch was kept closed for a long time before opening it at t=0. The

- voltage v(0+) is

- a) -10 V b) -1 V c) 0V d) 10 V

- 15.The input impedance of a series RLC circuit operating at frequency W=√2w, w being the resonant

- frequency, is

- a) R-j(wL/√2) ohms b) R+j(wL/√2) ohms c) R-j√2wL ohms d) R-j√2wL ohms

- 16.The threshold voltage V is negative for

- a) an n-channel enhancement MOSFET b) an n-channel depletion MOSFET c) an p-channel

- depletion MOSFET d) an p-channel JFET

- 17.At a given temperature, a semiconductor with intrinsic carrier concentration ni= 10 ^ 16 / m^3 is

- doped with a donor dopant of concentration Nd = 10 ^ 26 /m^3. Temperature remaining the same, the

- hole concentration in the doped semiconductor is

- a) 10 ^ 26 /m^3 b) 10 ^ 16 /m^3 c) 10 ^ 14 /m^3 d) 10 ^ 6 /m^3}

- 18.At room temperature, the diffusion and drift constants for holes in a P-type semiconductor were

- measured to be Dp = 10 cm^2/s and μp = 1200 cm^2/V-s, respectively. If the diffusion constant of

- electrons in an N-type semiconductor at the same temperature is Dn = 20 cm^2/s, the drift constant for

- electrons in it is

- a) μn = 2400 cm^2/V-s b) μn = 1200 cm^2/V-s c) μn = 1000 cm^2/V-s d) μn = 600 cm^2/V-s

- 19.A common LED is made up of

- a) intrinsic semiconductor b) direct semiconductor c) degenerate semiconductor d) indirect

- semiconductor

- 20.When operating as a voltage regulator, the breakdown in a Zener diode occurs due to the

- a) tunneling effect b) avalanche breakdown c) impact ionization d) excess heating of the junction.

- 21.If the common base DC current gain of a BJT is 0.98, its common emitter DC current gain is

- a) 51 b) 49 c) 1 d) 0.02

- 22.Negative resistance characteristics is exhibited by a

- a) Zener diode b) Schottky diode c) photo diode d) Tunnel diode

- 23.Let En and Ep, respectively, represent the effective Fermi levels for electrons and holes during

- current conduction in a semiconductor. For lasing to occur in a P-N junction of band-gap energy 1.2

- eV, (En - Ep) should be

- a) greater than 1.2eV b) less than 1.2eV c) equal to 1.1eV d) equal to 0.7eV

- 24.In a P-well fabrication process, the substrate is

- a) N-type semiconductor and is used to build P-channel MOSFET

- b) P-type semiconductor and is used to build P-channel MOSFET

- c) N-type semiconductor and is used to build N-channel MOSFET

- d) P-type semiconductor and is used to build N-channel MOSFET

- 25.In a MOS capacitor with n-type silicon substrate, the Fermi potential ¢ = -0.41 V and the flat-band

- voltage Vfb = 0V. The value of the threshold voltage Vt is

- a) -0.82 V b) -0.41 V c) 0.41 V d) 0.82

- Refer given figure for question 26 and 27. Assume D1 and D2 to be ideal diodes.

- 26.Which one of the following statements is true?

- a) Both D1 and D2 are ON.

- b) Both D1 and D2 are OFF.

- c) D1 is ON and D2 is OFF.

- d) D2 is ON and D1 is OFF.

- 27.Values of Vo and I, respectively, are

- a) 2V and 1.1 mA b) 0V and 0 mA c) -2V and 0.7 mA d) 4V and 1.3 mA

- 28.In a BJT CASCODE pair, a

- a) common emitter follows a common base

- b) common base follows a common collector

- c) common collector follows a common base

- d) common base follows a common emitter

- 29.Inside a 741 op-amp, the last functional block is a

- a) differential amplifier b) level shifter c) class-A power amplifier d) class-AB power amplifier

- 30.For the MOSFET in the given circuit, the threshold voltage Vt = 0.5V, the process parameter KP =

- 150 μA/V^2 and W/L = 10. The values of Vd and Id, respectively, are

- a) Vd = 4.5 V and Id = 1 mA

- b) Vd = 4.5 V and Id = 0.5 mA

- c) Vd = 4.8 V and Id = 0.4 mA

- d) Vd = 6 V and Id = 0 mA

- 31.A negative feedback is applied to an amplifier with the feedback voltage proportional to the output

- current. This feedback increases the

- a) input impedance of the amplifier b) output impedance of the amplifier c) distortion in the

- amplifier d) gain of the amplifier

- 32.The early effect in a BJT is modeled by the small signal parameter

- a) r0 b) rΠ c) gm d) β

- 33.For a given filter order, which one of the following type of filters has the least amount of ripple both

- in pass-band and stop-band?

- a) Chebyshev type I b) Bessel c) Chebyshev type II d) Elliptic

- 34.For a practical feedback circuit to have sustained oscillation, the most appropriate value of the loop

- gain T is

- a) 1 b) -1 c) -1.02 d) 1.02

- 35.Assume the op-amps in given figure to be ideal. If the input signal vi is a sinusoid of 2V peak-to peak and with zero DC component, the output signal vo is a

- a) sine wave b) square wave c) pulse train d) triangular wave

- 36.In a common source amplifier, the mid-band voltage gain is 40 dB and the upper cutoff frequency is

- 150kHz. Assuming single pole approximation for the amplifier the unity gain frequency fT is

- a) 6 MHz b) 15 MHz c) 150 MHz d) 1.5 GHz

- 37.An op-amp is ideal except for finite gain and CMRR. Given the open loop differential gain

- Ad=2000, CMRR = 1000, the input to the non inverting terminal is 5.002 V and the input to the

- inverting terminal is 4.999 V, the output voltage of the op-amp is

- a) 14 V b) 24 V c) -6 V c) -8 V

- 38.The op-amp in the circuit in given figure has a non-zero DC offset. The steady state value of the

- output voltage Vo is

- a) –RC dvs(t)/ dt b) – (1/RC)|vs(t)dt c) –V d) +V

- 39.For the circuit in given figure, if the value of the capacitor C is doubled, the duty-cycle of the output

- waveform Vo

- a) increases by a factor of 2 b) increases by a factor of 1.44 c) remains constant d)

- decreases by a factor of 1.44

- 40.Assume the op-amp in the given circuit to be ideal. The value of the output voltage Vo is

- a) 3.2 Vi b) 4 Vi c) 9 Vi d) 10 Vi

- 41.The complement of the Boolean expression F = (X + Y¯ + Z)(X¯ + Z¯)(X + Y) is

- a) XYZ+XZ¯+Y¯Z b) X¯YZ¯+XZ+X¯Y¯ c) X¯YZ¯+XZ+YZ d) XYZ+X¯Y¯

- 42.The Boolean function F(A,B,C,D) = Σ(0,6,8,13,14) with don’t care conditions d(A,B,C,D) =

- Σ(2,4,10) can be simplified to

- a) F = B¯D¯+CD¯+ABC¯ b) F = B¯D¯+CD¯+ABC¯D c) F = AB¯D¯+CD¯+ABC¯ d)

- F = B¯D¯+CD¯+ABCD

- 43.The Boolean function F = A¯D¯+B¯D can be realized by one of the following figures

- 44. For the multiplexer in given figure, the Boolean expression for the output Y is

- a) A¯B¯+B¯C¯+AC b) AB¯+B¯C¯+AC¯ c) AB¯+B¯C+AC d) A¯B¯+B¯C+A¯C

- 45. Which one of the following is TRUE?

- a) Both latch and flip-flop are edge triggered.

- b) A latch is level triggered and a flip-flop is edge triggered.

- c) A latch is edge triggered and a flip-flop is level triggered.

- d) Both latch and flip-flop are level triggered.

- 46. In a schottky TTL gate, the Schottky diode

- e) increases the propagation delay

- f) increases the power consumption

- g) prevents saturation of the output transistor

- h) keeps the transistor in cutoff region

- 47. For which one of the following ultraviolet light is used to erase the stored contents

- a) PROM b) EPROM c) EEPROM d) PLA

- 48. Which one of the following is NOT a synchronous counter

- a) Johnson counter b) Ring counter c) Ripple counter d) Up-down counter

- 49. In 8085 microprocessor, the accumulator is a

- a) 4 bit register b) 8 bit register c) 16 bit register d) 32 bit register

- 50. In the register indirect addressing mode of 8085 microprocessor, data is stored

- a) at the address contained in the register pair

- b) in the register pair

- c) in the accumulator

- d) in a fixed location of the memory

- 51. The output w[n] of the system shown in given figure is

- a) x[n] b) x[n-1] c) x[n] – x[n-1] d) 0.5(x[n-1] + x[n])

- 52. Which one of the following is a periodic signal

- a) x(t) = 2 e^j(t+(π/4)) b) x[n] = u[n] + u[-n] c) x[n] = Σ{∂[n-4k]-∂[n-1-4k]} where k = -∞to

- ∞ d) x(t) = e^ (-1+j)t

- 53. If the input-output relation of a system is y(t) = ∫x(t) dt where t = -∞ to 2t

- a) linear, time-invariant and unstable

- b) linear, non-causal and unstable

- c) linear, causal and time invariant

- d) non-causal, time invariant and unstable

- 54. Which one of the can be the magnitude of the transfer function | H(jw) | of a causal system

- 55. Consider the function H(jw) = H1(w) + jH2(w), where H1(w) is an odd function and H2(w) is an

- even function. The inverse Fourier transform of H(jw) is

- a) a real and odd function

- b) a complex function

- c) a purely imaginary function

- d) a purely imaginary and odd function

- 56. The laplace transform of given signal is

- a) –A((1-e^cs)/s) b) A((1-e^cs)/s) c) A((1-e^-cs)/s) d) –A((1-e^-cs)/s)

- 57. If X(z) is the z-transform of x[n] = (1/2)^ |n|, the ROC of X(z) is

- a) |z| > 2 b) |z| < 2 c) 0.5<|z|<2 d) the entire z-plane

- 58. In a linear phase system, τg the group delay and τp the phase delay are

- a) constant and equal to each other b) τg is a constant and τp is proportional to w c) a constant and τg

- is proportional to w d) τg is proportional to w and τp is proportional to w

- 59. A signal m(t), band-limited to a maximum frequency of 20 kHz is sampled at a frequency fs kHz to

- generate s(t). An ideal low pass filter having cut-off frequency 37 kHz is used to reconstruct m(t) from

- s(t). The maximum value of fs required to reconstruct m(t) without distortion is

- a) 20 kHz b) 40kHz c) 57 kHz d) 77 kHz

- 60. If the signal x(t) shown in given figure is fed to an LTI system having impulse response h(t) as

- shown in given figure, the value of the DC component present in the output y(t) is

- a) 1 b) 2 c) 3 d) 4

- 61. The characteristic equation of an LTI system is given as s^3 + Ks^2 + 5s + 10. When the system is

- marginally stable, the value of K and the sustained oscillation frequency w, respectively, are

- a) 2 and 5 b) 0.5 and √5 c) 0.5 and 5 d) 2 and √5

- 62. The time required for the response of a linear time-variant system to reach half the final value for

- the first time is

- a) delay time b) peak time c) rise time d) decay time

- 63. The signal flow graph of the given network is

- 64. Let c(t) be the unit step response of a system with transfer function K(s+a)/(s+K). If c(0+)=2 and

- c(∞)=10, then the values of a and K, respectively, are

- a) 2 and 10 b) -2 and 10 c) 10 and 2 d) 2 and -10

- 65. The loop transfer function of an LTI system is G(s)H(s)= K(s+1)(s+5) / s(s+2)(s+3). For K>0, the

- point on the real axis that DOES NOT belong to the root locus of the system is

- a) -0.5 b) -2.5 c) -3.5 d) -5.5

- 66. The state space equation of the circuit shown in given figure for x1=v0, x2=I is

- 67. The open loop gain of a unity feedback system is G(s)=wn^2 / s(s+2wn). The unit step response c(t)

- of the system is

- 68. If A = [2 0 0 2], then e^At is given by

- a) [e^2t 0 0 e^2t] b) [e^-2t 0 0 e^-2t] c) [e^t/2 0 0 e^t/2] d) [e^-t/2 0 0 e^-t/2]

- 69. The angles of the asymptotes of the root loci of the equation s^3 + 5s^2 + (K+2)s + K = 0, for

- 0<=K<∞, are

- a) 0 and 270 b) 0 and 180 c) 90 and 270 d) 90 and 180

- 70. The bode plot corresponding to a proportional derivative controller is the one shown in given figure

- 71. In frequency modulation, the instantaneous

- a) amplitude of the carrier signal is varied with the instantaneous amplitude of the message signal

- b) amplitude of the carrier signal is varied with the instantaneous frequency of the message signal

- c) frequency of the carrier signal is varied with the instantaneous amplitude of the message signal

- d) frequency of the carrier signal is varied with the instantaneous frequency of the message signal

- 72. If X is a zero mean Gaussian random variable, then P{X<=0} is

- a) 0 b) 0.25 c) 0.5 d) 1

- 73. If a single-tone amplitude modulated signal at a modulation depth of 100% transmits a total power

- of 15W, the power in the carrier component is

- a) 5W b) 10W c) 12W d) 15W

- 74. In a superheterodyne receiver, rejection of the image signal can be achieved by using a

- a) higher local oscillation frequency b) crystal oscillator c) narrow band IF filter d) narrow band

- filter at RF stage

- 75. The number of bbits per sample of a PCM system depends upon the

- a) sampler type b) quantizer type c) number of levels of the quantizer d) sampling rate

- 76. Which one of the following is used for the detection of AM-DSB-SC signal

- a) Ratio detector b) Foster-Seeley discriminator c) Product demodulator d) Balanced-slpoe

- detector

- 77. Which one of the following signal pairs can represent a BPSK signal

- a) A cos2πfct, A sinπfct

- b) A cos2πfct, - A sinπfct

- c) - A cos2πfct, A sinπfct

- d) A sin2πfct, A cosπfct

- 78. Which one of the following can be used for the detection of the noncoherent BPSK signal

- a) matched filter b) phase-locked loop c) envelope detector

- d) product demodulator

- 79. Bits of duration Tb are to be transmitted using a BPSK modulation with a carrier of frequency Fc

- Hz. The power spectral density of the transmitted signal has the first null at the normalized frequency

- a) |F – Fc|Tb = 0 b) |F – Fc|Tb = 1 c) |F – Fc|Tb = 2 d) |F – Fc|Tb = 4

- 80. The probability of bit error of a BPSK modulation scheme, with transmitted signal energy per bit

- Eb, in an additive white Gaussian noise channel having one-sided power spectral density N0, is

- a) (1/2) erfc(Eb/2N0) b) (1/2) erfc√(Eb/2N0) c) (1/2) erfc(Eb/N0) d) (1/2) erfc√ (Eb/N0)

- 81. For a given transmitted pulse p(t), 0<=t<=T, the impulse response of a filter matched to the

- received signal is

- a) –p(t-T), 0<=t<=T b) –p(T-t), 0<=t<=T c) p(t-T), 0<=t<=T d) p(T-t), 0<=t<=T

- 82. The multiple access communication scheme in which each user is allocated the full available

- channel spectrum for a specified duration of time is known as

- a) CDMA b) FDMA c) TDMA d) MC-CDMA

- 83. GSM system uses TDMA with

- a) 32 users per channel b) 16 users per channel c) 8 users per channel d) 4 users per channel

- 84. If Rx(τ) is the auto-correlation function of a zero-mean wide-sense stationary random process X,

- then which one of the following is NOT true?

- a) Rx(τ) = Rx(-τ) b) Rx(τ) = -Rx(-τ) c) σx^2 = Rx(0) d) |Rx(τ)| <=Rx(0)

- 85. If E denotes the expectation operator, then E[X-EX]^3 of a random variable X is

- a) EX^3 – E^3X b) EX^3 + 2E^3X – 3EX Ex^2 c) 3EX^3 – E^3X d) 2EX^3 + E^3X –

- 3EX EX^2

- 86. A discrete memoryless source produces symbols m1,m2,m3 and m4 with probabilities 1/2, 1/4 , 1/8

- and 1/8, respectively. The entropy of the source is

- a) ¼ b) 1 c) 7/4 d) 2

- 87. A channel has a signal-to-noise ratio of 63 and bandwidth of 1200 Hz. The maximum data rate that

- can be sent through the channel with arbitrary low probability of error is

- a) 600 bps b) 1200 bps c) 4800 bps d) 7200 bps

- 88. For the vectors A = X ax + Y ay and B = Z az, del . (A X B) is

- a) 0 b) 1 c) XZ d) YZ

- 89. Which one of the following relations represents Strokes’ theorem (symbols have their usual

- meaning)?

- a) ∫s del X A.ds = 0 b) ∫L A.dl = ∫s del X A.ds c) ∫s A X dS = -∫v (del X A)dv d) ∫v del.Adv

- = ∫s A.ds

- 90. Which one of the following relations is not correct (symbols have their usual meaning)?

- a) del X E = - ∂B/∂t b) del X H = J + ∂E/∂t c) del.D = ρv d) del.B = 0

- 91. The electric field component of a uniform plane wave propagating in a lossless magnetic dielectric

- medium is given by E(t,z)=ax 5cos(10^9 t – 20/3 z)V/m. If η0 represents the intrinsic impedance of the

- free space, the corresponding magnetic field component is given by

- a) H(t,z)= ay 5/2 η0 cos(10^9t – 20/3 z)A/m

- b) H(t,z)= ay 10/ η0 cos(10^9t – 20/3 z)A/m

- c) H(t,z)= az 5/2 η0 cos(10^9t – 20/3 z)A/m

- d) H(t,z)= az 10/ η0 cos(10^9t – 20/3 z)A/m

- 92. The skin depth of a non-magnetic conducting material at 100 MHz is 0.15 mm. The distance which

- a plane wave of frequency 10 GHz travels in this material before its amplitude reduces by a factor of

- e^-1 is

- a) 0.0015 mm b) 0.015 mm c) 0.15 mm d) 1.5 mm

- 93. A lossless transmission line has a characteristic impedance of 100 ohms and an inductance per unit

- length of 1 μH/m. If the line is operated at 1 GHz, the propagation constant β is

- a) 2π rad/m b) 20π/3 rad/m c) 20π rad/m d) 2π *10^5 rad/m

- 94. When a load resistance Rl is connected to a lossless transmission line of characteristic impedance

- 75 ohms, it results in a VSWR of 2. The load resistance is

- a) 100 ohms b) 75√2 ohms c) 120 ohms d) 150 ohms

- 95. A two-port network characterized by the S-parameter matrix, [S] = [0.3 L0 0.9 L90

- 0.9 L90 0.2 L0] Is

- a) both reciprocal and lossless b) reciprocal, but not lossless c) lossless, but not reciprocal d)

- neither reciprocal nor lossless

- 96. A lossless air filled rectangular waveguide has internal dimensions of a cm * b cm. If a=2b and the

- cutoff frequency of the TE02 mode is 12 GHz, the cutoff frequency of the dominant mode is

- a) 1 GHz b) 3 GHz c) 6 GHz d) 9 GHz

- 97. A Hertzian dipole antenna is placed at the origin of a coordinate system and it is oriented along zaxis. In which one of the following planes the radiation pattern of the antenna has a circular shape?

- a) x=0 b) y=0 c) z=0 d) ø=45

- 98. Which one of the following statements is not true?

- a) Antenna losses are taken into account in calculating its power gain

- b) For an antenna which does not dissipate any power, the directive gain and the power gain are equal

- c) Directivity of an antenna is the maximum value of its directive gain

- d) The directive gain of a Hertzian dipole is same in all direction

- 99. The directivity of a half dipole antenna is

- a) 1.0 b) 1.5 c) 1.64 d) 2

- 100. Which one of the following is not true for a step index optical fibre?

- a) It can support multiple modes

- b) HE11 mode is its lowest order mode

- c) The refractive index of the cladding is higher than that of the core

- d) At a given wavelength, single mode operation is possible by proper choice of core diameter, core

- and cladding refractive indices.

- SECTION – B

- 101. Sarnath is situated in the state of

- a) MP b) Bihar c) Punjab d) UP

- 102. Green house effect is due to the increase of atmospheric

- a) CO2 level b) SO2 level c) CO level d) N2 level

- 103. In the month of July, it is winter in

- a) New York b) Beijing c) Sydney d) London

- 104. The chairman of the Planning commission of India is

- a) The prime minister b) The vice-president c) The union finance minister d) The union

- commerce minister

- 105. The satellite launch vehicle that placed a number of satellites ito orbit in May 2008 is

- a) PSLV-C7 b) PSLV-C8 c) PSLV-C9 d) PSLV-C10

- 106.DRDO was formed in

- a) 1947 b) 1950 c) 1954 d) 1958

- 107. SAMYUKTA is developed for the use of

- a) Navy b) Army c) Air force d) RAC

- 108. DARL 202 is a variety of

- a) pea b) garlic c) capsicum d) tomato

- 109. TRISHUL is

- a) a surface to surface battlefield missile

- b) a quick reaction surface to air missile

- c) an intermediate range ballistic missile

- d) a supersonic cruise missile

- 110. HUMSA is a

- a) sonar b) tank c) mine d) night vision device

- 111. The value of 1+2i / 3-4i + 2-I / 5i , where i^2 is -1, is

- a) -5/2 b) 5/2 c) 2/5 d) -2/5

- 112. The particular solution of the differential equation d^2y/dx^2 + 2 dy/dx + 5y = 0 satisfying the

- conditions y(0)=0 and y’(0)=1 is

- a) y=1/2 e^-x cos2x b) y=1/2 e^-x sin4x c) y=1/2 e^-x sin2x d) y=1/2 e^-x cos4x

- 113. For the vectors A=3i-2j+k and B=2i-k, the value of (A*B).A is

- a) 0 b) 1 c) 2 d) 3

- 114. The orthogonal trajectory of the family of curves x^2-y^2 = a (where a is a constant) and passing

- through the point (1,1) is

- a) y=-1/x b) y=1/x c) y=-x d) y=x

- 115. The value of the line integral ∫ y^2 dx + 2xydy over the curve x=accost, y=asint is

- a) 0 b) 1 c) 2 d) 4

- 116. The n-th partial sum of the infinite series 1/1*2 + 1/2*3 + 1/3*4+……1/n*(n+1)……..

- a) 1/n+1 b) n+2/n+1 c) n/n+1 d) n-1/n+1

- 117. The complex-valued function f(z)=e^z is analytic for

- a) no z b) all z c) real z only d) imaginary z only

- 118. The inverse of the matrix [ cos A sin A

- -sin A cos A] is

- a) [ -cos A Sin A b) [cos A sin A} c) [cos A -sin A d) [cos A -sin A

- sin A cos A] sin A -cos A] -sin A cos A] sin A cos A]

- 119. Consider the function f(x) defined as

- F(x) = 3x-1, x<0

- 0, x=0

- 2x+5, x>0

- In the following table, List I shows 4 expressions for limits of f(x) and List II indicates the values of the

- limits

- List I List II

- P.Lim x->2 f(x) 1. -1

- Q.Lim x->0+ f(x) 2. 9

- R.Lim x->0- f(x) 3. -10

- S.Lim x->-3 f(x) 4. 5

- The correct matches are

- a) P-2,Q-4,R-1,S-3 B) P-2,Q-4,R-3,S-1 C) P-4,Q-2,R-1,S-3 D) P-4,Q-2,R-3,S-1

- 120. Two events A and B with probability 0.5 and 0.7, respectively, have joint probability of 0.4. The

- probability that neither A nor B happens is

- a) 0.2 b) 0.4 c) 0.6 d) 0.8

- 121. Consider the differential equation

- X^2 d^2/dx^2 + x dy/dx + (x^2 - 4)y = 0. The statement which is not true for it is

- a) It is a linear second order ordinary differential equation

- b) It can not be reduced to a differential equation with constant coefficients

- c) X=0 is a regular singular point

- d) It is a non-homogeneous second order ordinary differential equation

- 122. The sum of two numbers is 16 and the sum of their squares is a minimum. The two numbers are

- a) 10,6 b) 9,7 c) 8,8 d) 5,11

- 123. The value of the definite integral 0∫(π/2)^(1/3) x^2 sin(x^3)dx is

- a) -1/3 b) 0 c) 1 d) 1/3

- 124. A circle C2 is concentric with the circle C1 : x^2 + y^2 -4x +6y -12 =0 and has a radius twice that

- of C1. The equation of the circle C2 is

- a) x^2 + y^2 -4x +6y -13 =0 b) x^2 + y^2 -4x +6y -87 =0 c) x^2 + y^2 -4x +6y -100 =0 d)

- x^2 + y^2 -4x +6y -88 =0

- 125. Consider the quadratic equation x^2 + px + q =0. If p and q are roots of the equation, the values of

- p and q are

- a) p=0, q=0 only b) p=1, q=-2 only c) p=0, q=0 and p=1, q=-2 d) p=0, q=0 and

- p=-2, q=1

- 126. Consider the list of words: etiquette, accommodate, forty, exaggerate, continous, independent,

- receipt. The number of misspelt words are

- a) 1 b) 2 c) 3 d) 4

- 127. Consider the following sentences

- 1. A few friends he has are all very rich.

- 2. Do not insult the weak.

- 3. The later of the two persons was more interesting.

- 4. All the informations were correct.

- Out of these sentences, the grammatically correct sentence is

- a) 1 b) 2 c) 3 d) 4

- 128. The appropriate auxiliary verb to fill in the blank of the sentence “Gandhi knew that he __ soon be

- jailed.”is

- a) would b) will c) shall d) may

- 129. The number of missing punctuation marks in the sentence “Rajesh along with Amit went to the

- market.”is

- a) 0 b) 1 c) 2 d) 3

- 130. The meaning of the word PLAGIARISM is

- a) theft of public money b) theft of ideas c) belief in one god d) belief in many gods

- 131. The antonym of the word TRANSIENT is

- a) certain b) close c) permanent d) fast

- 132. ACROPHOBIA is the abnormal fear of

- a) open spaces b) height c) fire d) water

- 133. The appropriate pair of prepositions to fill in the blank in the sentence “He was angry __ me,

- because my remarks were aimed __ him.”is

- a) at,to b) with, at c) with, to d) at, for

- 134. The appropriate word(s) to fill up the blank in the sentence “ I remember __ voices in the middle

- of the night.”is (are)

- a) hear b) to hear c) hearing d) heard

- 135. The passive voice form of the sentence “I have known him for a long time.”is

- a) He is known to me for a long time.

- b) He is known by me for a long time.

- c) He has been known to me for a long time.

- d) He has been known by me for a long time.

- 136. If kennel is to a dog, then __ is to a hen.

- a) nest b) coop c) hole d) stable

- 137. If NATION is to 5236765, then NOTION is to

- a) 573675 b) 563765 c) 576375 d) 557365

- 138. The next two numbers of the series 3,5,11,21 are

- a) 34 and 52 b) 34 and 53 c) 35 and 52 d) 35 and 53

- 139. A, B and C are three places in India with longitudes 80E, 85 E and 90 E respectively. Which one

- of the following statements about the local times of the places is true?

- a) Local time of C is ahead of that of B.

- b) Local time of B is ahead of that of C.

- c) Local time of A is ahead of that of C.

- d) A, B and C all have the same local time.

- 140. In this question, notations +, / and * are used as follows

- A + B means A is the husband of B.

- A / B means A is the sister of B.

- A * B means A is the son of B.

- With these relations, the relationship denoted by P / Q * R is

- a) P is son of R

- b) P is daughter of R

- c) P is uncle of R

- P is father of R

- 141. If DELHI is written as EDHIL, then PARIS is written as

- a) APRIS b) SARIP c) SAPIR d) APISR

- 142. The number of prime numbers between 10 and 50 is

- a) 10 b) 11 c) 12 d) 13

- 143. The odd one in the list : LAN, TCP/IP, HACKER and KILLER is

- a) LAN b) TCP/IP c) KILLER d) HACKER

- 144. SAW is to carpenter as SCALPEL is to

- a) surgeon b) mason c) plumber d) tailor

- All the best!!

- Shilpa

- DRDO Aptitude Paper 2008

- DRDO PAPER ON 12th SEPTEMBER 2008

- Paper Pattern

- 1. Miss Universe 2008 is—

- (A) Riyo Mori

- (B) Dayana Mendoza

- (C) Lisa Lazarus

- (D) Simaran Kaur Mundii

- (E) None of these

- 2. Who among the following has awarded UNESCO / Guillermo Cano world press freedom award for

- 2008 ?

- (A) Anna Politkovskaya

- (B) Amira Hass

- (C) Raul Rivero

- (D) Lydia Cacho Rebeiro

- (E) None of these

- 3. The present Governor of Bihar is—

- (A) R.L.Bhatia

- (B) R.S.Gavai

- (C) Buta Singh

- (D) S.S.Sidhu

- (E) None of these

- 4. India test-fired Nuclear Capable Agni III on May 7, 2008 for—

- (A) Fourth time

- (B) Second time

- (C) Third time

- (D) Fifth time

- (E) None of these

- 5. Tenth century coins have been found from the city of—

- (A) Guwahati

- (B) Bareily

- (C) Khandwa

- (D) Ranchi

- (E) None of these

- 6. The cyclone which struck Myanmar in May 2008 was—

- (A) Willy-willy

- (B) Nargis

- (C) Fanoos

- (D) Nasim

- (E) None of these

- 7. The number of children, who are deprived of basic healthcare in India, as per the report—state of the

- World’s Mothers—brought out by global humanitarian organization ‘save the children’ is—

- (A) 60 million

- (B) 50 million

- (C) 67 million

- (D) 55 million

- (E) None of these

- 8. The country where the system of two Supreme Courts has been proposed is—

- (A) Pakistan

- (B) Nepal

- (C) Sri Lanka

- (D) Bhutan

- (E) None of these

- 9. Which of the following is not a member of the NATO ?

- (A) Austria

- (B) Poland

- (C) Hungary

- (D) Spain

- (E) None of these

- 10. Who became Britain’s first Asian woman Lord Mayor ?

- (A) Rekha Rani

- (B) Anjula Sood

- (C) Reshma Shah

- (D) Kavita Sahni

- (E) None of these

- 11. The newly elected President of Paragury is—

- (A) Fernando De Silva

- (B) Fernando Lugo

- (C) Fernando Silvio

- (D) Fernando Dominique

- (E) None of these

- 12. Silvio Berlusconi was recently elected to the post of Prime Ministership of Italy. He belongs to—

- (A) Italian Nationalist Party

- (B) The Patriotic Alliance

- (C) People’s Party of Italy

- (D) People of Freedom Party

- (E) None of these

- 13. PLA stands for—

- (A) Party of Leninist Association

- (B) People’s Liberation Army

- (C) People for Liberal Alliance

- (D) People for Left Alliance

- (E) None of these

- 14. The number of African countries which attended the recently concluded India-Africa Forum

- Summit is—

- (A) 14

- (B) 15

- (C) 16

- (D) 17

- (E) None of these

- 15. According to a recent survey which of India’s states has topped in rural electrification ?

- (A) Maharashtra

- (B) Tamil Nadu

- (C) Rajasthan

- (D) Karnataka

- (E) None of these

- 16. Simran Kaur Mundi has been chosen—

- (A) Miss India Earth 2008

- (B) Miss India Universe 2008

- (C) Miss India World 2008

- (D) None of the above

- (E) None of these

- 17. Which one of the following won Jawaharlal Nehru Award for International Understanding for 2007

- ?

- (A) Olafur Ragnar

- (B) Javed Akhtar

- (C) Luiz Lula da Silva

- (D) The Milinda and Gates Foundation

- (E) None of these

- 18. The book ‘Unaccustomed Earth’ is written by—

- (A) P. M. Nayyar

- (B) Shobha De

- (C) Jhumpi Lahiri

- (D) None of these

- (E) None of these

- 19. The 2008 Edition of Wisden Cricketers’ Almanack–Crickets major annual reference work has

- named “The Leading Cricketer of the World for 2007”—

- (A) Mc Grath

- (B) Jacques Kallis

- (C) Ricky Ponting

- (D) Sachin Tendulkar

- (E) None of these

- 20. India ranks …………… in foreign exchange forex reserve.

- (A) Third

- (B) Second

- (C) Fifth

- (D) Fourth

- (E) None of these

- 21. Raghuram Rajan Committee is related to—

- (A) Austerity in government expenditure

- (B) Study of causes of rising prices

- (C) Financial sector reforms

- (D) Export-import balance

- (E) None of these

- 22. The International Monetary Fund has estimated India’s contribution to World Gross Domestic

- Product in Purchasing Power Parity (PPP) terms for 2007 to—

- (A) 4·6%

- (B) 6·4%

- (C) 7·1%

- (D) 3·9%

- (E) None of these

- 23. According to data released by the Ministry of Finance, currently the per capita debt on each of the

- roughly 110 crore Indians stands at—

- (A) Rs. 71502

- (B) Rs. 7218

- (C) Rs. 10550

- (D) Rs. 6103

- (E) None of these

- 24. Which one of the three single cross improved hybrids of maize has not been developed and

- recommended for an All India level use by Chaudhry Charan Singh Haryana Agriculture University ?

- (A) HPM–6

- (B) HM–8

- (C) HM–10

- (D) HQPM–7

- (E) None of these

- 25. US has decided to remove ………from its list of states sponsoring terrorism.

- (A) Cuba

- (B) Iraq

- (C) North Korea

- (D) Libya

- (E) None of these

- 26. In the year 2006-07 India received Foreign Direct Investment Worth—

- (A) $ 25 billion

- (B) $ 24·5 billion

- (C) $ 26·25 billion

- (D) 20·5 billion

- (E) None of these

- 27. For the first time, the T.V. Cameras and print cameramen were allowed to record proceedings of

- the Lok Adalats dispensing justice on—

- (A) May 3, 2008

- (B) May 5, 2008

- (C) May 7, 2008

- (D) May 4, 2008

- (E) None of these

- 28. As a part of the partial solution to the world’s energy crisis, the scientists of which of the following

- countries are building a nuclear fusion laboratory designed to recreate the temperatures and pressure of

- an exploding star ?

- (A) U.K.

- (B) U.S.A.

- (C) Australia

- (D) France

- (E) None of these

- 29. Recently, a fatal disease broke out in China infecting thousands of children and causing death of

- some of them. The disease was—

- (A) Bird flu

- (B) Filariasis

- (C) Hand, mouth and foot disease

- (D) Chicken pox

- (E) None of these

- 30. Two Indian doctors from Kolhapur have achieved a path breaking success in the research of—

- (A) Genetics

- (B) Stem cell

- (C) New protein structure

- (D) Habit pattern of different people

- (E) None of these

- 31. Apart from ISRO, three other agencies/institutions were also involved in the execution of PSLVC9.

- Which of the following was not involved ?

- (A) DRDO

- (B) Antrix Corporation

- (C) Cosmos International Germany

- (D) University of Toronto, Canada

- (E) None of these

- 32. The Iran-Pakistan-India gas pipeline, slated to begin in 2009 was conceptualised first in—

- (A) 1991

- (B) 1989

- (C) 1985

- (D) 1993

- (E) None of these

- 33. In addition to Kolkata and Delhi, the Union Government plans to start Metro rail service in four

- more cities. Which of the following is not included in the plan ?

- (A) Kochi

- (B) Hyderabad

- (C) Pune

- (D) Chennai

- (E) None of these

- 34. President Pratibha Patil recently visited Chile, Brazil and Mexico. During her trip, with which of

- the following countries did India sign an extradition treaty—

- (A) Brazil

- (B) Chile

- (C) Mexico

- (D) None of these

- (E) None of these

- 35. The winner of the 2008 Wimbledon Open Tennis Tournament men's single title is -

- (A) Roger Federer

- (B) Andy Roddick

- (C) Novak Djokovic

- (D) Rafael Nadal

- (E) None of these